Basic Abu Reel Cleaning 101 For Newer Reels!!

Following is a basic step-by-step outline of breaking down; cleaning, lubing and reassembly of newer Abu Garcia bait casters with instant anti-reverse.

Before you get started, make sure you have the following tools and materials so you do not have to move until the reel is complete.  Roll of paper towels, Abu Oil (my preference), Abu Grease (my preference), screwdrivers of various sizes and styles, pointed awl type tool, pliers, smaller needle nose, 18mm wrench (I believe that is the size) and a crescent wrench.  Also a can of Reel Magic comes in handy sometimes.  One sided razor blade and a toothbrush is also necessary.

First get your work area ready.  Place a paper towel down in front of you as a workstation.  Make sure that you are working in a well lit area just incase any small parts fall or go flying.

Now it is time for the actual work.

- Remove the handle nut lock or nut cap.  Both would have a small screw that goes into the handle.

- Remove the handle nut.

- CAREFULLY remove the C-clip that is on the top of the shaft.  Without this off, the handle will not come off.  WARNING: IF NOT REMOVED CAREFULLY, THE C-CLIP HAS A FLYING DISTANCE OF ¼ MILE.  OKAY, SO IT CAN GO ACROSS THE ROOM, YOU GET THE POINT.

- Remove the handle and the handle washer (thin curved metal washer between handle and drag star.

- Remove drag star by turning back words and completely remove it. Keep all of the items removed so far together in one spot.

- Thoroughly wipe down all parts removed so far with a paper towel and keep in one spot.

- Loosen the three thumbscrews on the handle side plate and remove the whole plate assembly.  (They will not come out of the plate)

- Remove the spool and set aside

- Remove the axle from the side plate if it is not out already.  If it appears to be stuck tight, pull firmly with a pliers and it will come out.

- Remove the 2 screws on the handle side plate.

- Carefully remove the side plate and wipe ALL dirt, grease, oil, and grime from it.  If necessary, spray a little Reel Magic on a paper towel and wipe the side plates off getting in all grooves on the sides.  Use a paper towel to wipe out the A/R bearing that is permanently in the side plate if it looks to have excess oil/grease on it.

- Remove the drive shaft that has the main gear on it and take the main gear off.  If you have a newer reel with multiple drag washers, there should be no reason what so ever to replace the drag washers.

- Wipe the main gear, black drag washers and the silver metal washers down well.  They should not be oily or greasy when put back together.

- Place one black washer on the shaft, then the main gear, then a black washer, then the round, silver washer, then a black washer, then a silver washer with 2 nipples, then a black washer then the top plate.

- On top of the last washer, place the silver cylinder with the square inside down.  (Look at both ends and you will see what I mean)

- Take off the black position holder that has 2 thin metal arms.

- Take off the pinion gear and its black holder.  Pop the gear out and wipe both down well.

- Take the clutch arm off and wipe off.  This is one part that can wear.  If you are having problems with the button not popping back out, this is probably the culprit.  On the end that would be by the drive shaft there is a piece that sticks up.  If it is showing brass and is rounded, it should be replaced.  Inexpensive part.

- Take the push button off and wipe off.

- Take the bottom arm off and wipe off.

- Wipe down the main plate of all oil and grease

- Place the bottom arm back on

- Now the push button

- Now the clutch arm

- Now the pinion gear and holder.  Gear part up, beveled edge of holder down.

- Place a small amount of OIL on the main post and then replace the drive shaft/main gear assembly

- Take the grease and place a minimal amount of grease all the way around the main gear; too much is not good.

- Replace the black position holder and make sure it snaps all the way down to the plate.

- Place the side cover back on and put the 2 screws back in.

- Place the 2 curved washers back on the drive shaft so they are curved towards each other and will spring.

- Screw the star wheel back down

- Replace the thin curved washer on the star curved down

- Replace the handle

- Carefully snap the c-clip back on

- Put the nut on

- Put the nut lock or cap (which ever your reel has) back on and set assembly aside

- Remove 3 screws from palming side of reel

- Remove side cover and wipe the inside the plate of any grease or oil.  The plastic cog should also be wiped down well.  If the cogwheel is held on with a c-clip, remove the clip and take the cog off and wipe down well.  Place a drop of oil on the post and replace the cog and clip.  If the cog does not have a clip, wipe it down well and place a couple drops of oil on the top of the cog.

- If the reel has a clicker, remove the 2 rubber/plastic tall spacers that go on top of the clicker plate over the posts of the frame

- Remove the clicker “moon” shape plate

- Using a screwdriver, take the pawl cap off of the line guide on the frame and shake the pawl out.

- On the frame on the palming side there is a thin, long lock that slides and locks onto 2 posts on the frame.  Remove this

- Slide the worm gear out

- Remove the worm gear cover and the line guide

- Wipe down all parts very, very well.  Make sure that the inside of the worm gear cover is cleaned out well.

- ALL grease and oil should be gone off of the worm gear.

- If your reel has a plastic thumb rest, pop this off and brush down well.  You will probably see a lot of grime, dirt and weeds built up on the underside.

- Using the toothbrush, clean out the cross post of the frame that the line guide runs in, this is a trouble spot for people.

- Slide the worm gear back into the cover and through the frame

- Slide the line guide on making sure the top of the guide is running in the frame

- Replace the locking clip that holds the worm gear in-place

- Place the “moon” shaped clicker assembly on with the clicker are pointing towards the middle

- Place the 2-plastic/rubber spacers over the posts that the clicker is on.

- Replace the side plate and 3 screws

- Replace the spool

- Place only a single drop of oil on each of the 2 brake blocks and a drop or two of oil on the bearings in the end of the spool.

- Place the axle in the handle side of the plate and press down firmly on the table or hard surface until it clicks in

- Run a bead of OIL down the axle and replace in the spool.  Slide the side plate onto the posts of the frame for fitting and tighten the thumb screws

- Take the pawl and place it back into its proper position on the underside of the line guide.  Make sure that it is properly in-place and not sticking out of the bottom.

- Screw the pawl cap back on.

- Run a bead of OIL, NOT GREASE, across the worm gear.  Turn handle several times to get he line guide to move back and forth several times to work the oil in.

This is the quick run down of taking apart a reel, cleaning; lubing and getting it back together.  There should not be any parts left over.