sworrall: MuskieFIRST welcomes Luke Morris of Lowrance Electronics!!

lowrance_luke: evening folks, had a great time here last year, hope I can be as helpful this year.

sworrall: Let the questions begin

Trophymuskie: My new LMS320 is supposed to have dual frequency 50 and 200, i am not getting the 50 you know why? am i missing a transducer?

lowrance_luke: Dual frequency output requires the dual frequency transducer make sure your transducer is about the size of the palm of your hand and it should be a dual frequency model

Trophymuskie: I know they did not install the temp/speed gage as well. I guess they figured I did not need it with GPS but I do can I get one after market and install it?

Hoosierdaddy: Never installed a sounder/GPS before. Are there any tips you can give me for when the unit finally arrives.  I am more anxious then when my son was born

Trophymuskie: the transducer is regular size like my X85 had

lowrance_luke: primary to sonar performance is transducer location. Ensure you place it in a spot where it will remain in the water at all times and away from any cavitation or turbulence developed by your hull

essox2: Is there any way to run 2 of the same locators off of the same traducer ? or do i need a separate transducer for each

lowrance_luke: The X-85 standard transducer is a single frequency only

lowrance_luke: to run two sounders from 1 xdcr you will need a switch, also running two different transducers from a single sounder requires a switch

essox2: does lowrance sell these switch's ??

lowrance_luke: boy you guys are being nice and taking turns tonight........

Shane: What wattage is recommended for fishing lakes.  I know the higher the wattage the more detail....

lowrance_luke: You can obtain these by clicking onto www.lei-extras.com or calling the toll free number

Shady: Luke- Is the LMS 320 still available?

lowrance_luke: for most fresh water applications any sounder over 1200-1500 watts will provide good overall performance for most freshwater fishing applications

StephenDawg: Does Lowrance deal in reconditioned units? ( Say an X-15 for around $150...//1)

lowrance_luke: The 320 model was replaced this year with a 480 version, some 320's may be available on closeout from larger retailers. We do of course honor the 1 year warranty and will continue to provide repair services fro as long as possible

Hoosierdaddy: Why has Lowrance stayed away from building internal GPS antennas?

lowrance_luke: we do not retail our units and do not maintain an inventory of refurbished models for direct sales. All units in the factory are used to provide service to our customers until they are completely gone

essox2: My new boat comes with an X-47 . Is this a lower end unit or an average unit this is my first lowrance?

lowrance_luke: Introduced this year is the Eagle Cuda 240, Lowrance M52, and M56 models. Entry level sonar gps combos with internal GPS antennas

Slamr: Luke (Andrew here) if you had say $500 to spend on a sonar unit, and wanted it for water under 50ft deep, GPS and color don’t matter, and speed and temp aren’t an issue...what would you recommend and why?

lowrance_luke: The X-47 is an entry level lowrance model sounder

sparky: I have an X-47 on the front of my boat hooked up to the trolling motor battery.  Will I lose too much power on the trolling motor battery?  Should I hook it up to the starting battery?

lowrance_luke: Best bang for the buck in pure on the water performance for the price is the Eagle FishMark 480 model. full power, 480 vertical pix's and just under $200

troyz: What is the big difference between Lowrance and eagle, are they the same technology on equivalent units

Slamr: really....so for $300 less, I’m really not missing that much?

essox2: will there be a rep at the Milwaukee sport show this year

Shady: Is there a GPS/Sonar model under $500.00?

lowrance_luke: Eagle and lowrance are basically equivalent in performance. With the Lowrance model you will get some software functions and features, and accessory options not available in the Eagle product line.

lowrance_luke: Slamr for $300 less you only miss options and accessory choices.

lowrance_luke: The Milwaukee show is scheduled with the rep group covering that region. They will schedule attendance and I am not aware if they will be there or not.

Slamr: not power or clarity on the screen?

lowrance_luke: There are some higher power options like X-135 that are not available in the Eagle models but they   have the same LCD screens

Shane: How accurate are the speed and temp readings?  ie, +/- 3mph or +/- 3 degrees.....

Trophymuskie: Will the color TV (104C) come down in price after a year or two like most new electronics?

essox2: seems that lowrance out did hummingbird this year in the tracker new boats. I see most of their boats coming with lowrance on now

lowrance_luke: Using the trolling speed sensor, keyword trolling speed, it is accurate to +/- 2 tenths mph and temperature is  +/- 1 degree spec. They are usually more accurate though.... averaging 1 or 2 tenths

H_K_: Are any of the new able to scroll and read accurately at high speeds

muskie_man: hi luke. is the new ifinder pro gps ideal for fishing. for example can I find my way back on lake of the woods without buying one of those map cards?

lowrance_luke: The primary expense of the 104 model is the display, as the prices of components go down we will be able to pass these savings on to our customers. This year the LCX-19c and some others did come down

Shane: What transducer cone angle are most guys using for Muskie fishing?

lowrance_luke: We did pick up a couple OEM's. I guess they began to listen to there consumers or got tired of repalcing sounders on new ordered boats before pickup

Trophymuskie: well the display (10 inch I believe) is half the reason i love the unit, the other is the ability to see bottom in #D mode with it's color function

StephenDawg: While motoring across the lake I switch to "Fastrak" (Eagle Ultra Classic) to get an instantaneous reading. Is that correct logic? Are there other benefits of Fastrak?

lowrance_luke: The ifinder pro has a level of built in mapping without the detail cards and at worst can be used as a plotter to get you back to the ramp after a long day

Slamr: what is the advantage to slower or faster chart speed?

troyz: I am looking at the Lowrance 4800 GPS vs. Eagle 480 GPS only unit, what is the main difference between them

muskie_man: so you really don’t need the map cards.

lowrance_luke: cone angles are important only for the type for the objects you are looking for. In general the standard 20 cone provides up to 60 of coverage or visibility under the transducer and is generally the best options for most fresh water applications

Plitz: lowrance_luke: what would you say the best unit would be for say...2-3 hundred

lowrance_luke: Fastrack is real-time sonar returns. As far as real time goes, our latest digital sonar devices are actually faster and more real-time than flasher models built as little as 10 years ago

lowrance_luke: Difference in Lowrance and eagle GPS combos is mode functions, features, and screens on the lowrance and additional transducer and sensor accessory options.

ghoti: How do the color screens compare to the non color as far as visibility in full sunlight?

Trophymuskie: I like to use fastrack when I use a split screen, it lets me see thing coming in better in the shorter screen

StephenDawg: Fastrak of any value with boat at rest?

lowrance_luke: Slower chart speed sill compress like sonar returns and make them more distinguishable recommended for still fishing or jigging when normally alot of flat lines appear.

ToddM: How does the color flasher ice unit compare to the vexilar? (sorry if it's been asked already)

lowrance_luke: Faster for faster speds across the lake. This includes the ping speed adjustment which

allows the sounder to operate at over 30 pings per second with the same rate of chart speed

lowrance_luke: Best unit for 2-3 c-notes Eagles FishMark 480 model

muskie_man: Can the ifinder pro show you what the street name that you are on without the cards? Thanks

lowrance_luke: Fastrack at rest?.....especially when ice fishing to see really quick the action of baits, and your actions on fish as they occur

Shane: Is there any plan to make a "forward looking" transducer.  I'm wondering because we now see what is directly under us and not what dangers lay ahead.

lowrance_luke: Ice machine versus Vexilar...I took an Ice machine to Detroit for Christmas... unfortunately no ice.... reserve judgement. In general I like chart over flasher as a flasher provides echoes out of context, A chart provide real-time sonar returns on context with other returns to make it easier to read

StephenDawg: Shane: Tilting your transducer (on the trolling motor)  forward a little is an idea I've heard of. Luke, what do you think?

ToddM: thanks luke, some good info here on ice fishing without using a flasher

Shane: How about when motoring across a lake though?

lowrance_luke: Street level mapping details are only contained in the detail mapping and require the maps and card

lowrance_luke: For forward looking you require a specialized transducer which shoots a pencil thin beam. This technology is available but prohibitively priced for the recreational market. Units of this type are manufactured and start at several thousand dollars

troyz: With advancement in technology, do you for see GPS be used control kicker motor run routes that are saved on GPS, or saving a route on a GPS, and have your trolling motor follow the route

lowrance_luke: tilting a transducer will provide some forward looking but the unit will still track only bottom and anything closer than bottom directly below the transducer

lowrance_luke: miss anything.....

StephenDawg: Good clarification on the "Tilting" thing. Thanks

Shane: Thanks luke, great info, I'm learning lots!

ToddM: are there any differences in the universal sonar units in the minnkota vs the lowrance units?

lowrance_luke: Of course our transducers are designed specifically for our sounder and will provide optimum performance. Minkota took the initiative in developing universal to reduce problems between sounders and trolling motors we are all aware of. We supported Minkota in this effort and I assure you our sounders will perform at least as well and usually better than any other manufacturers on the Minnkota transducer. Performance is based on sounder capability and least of all transducer technology which is pretty even across manufacturers

H_K_: Is there a unit I should look for that has a frequency that is compatible with my eagle classic, or doesn’t it matter?

jlong: Luke, is the gossip true that Lowrance will be introducing a waterproof iFinder Pro this spring?

lowrance_luke: H_K_ Define compatible with eagle classic ie. using at the sa,e time or repalcing using same cabling and transducer

H_K_: with interferance with each other

lowrance_luke: The iFInder H20 is nearing full production and will soon be at retailers. Waterproof and fully mapping and navionics compatible.

jlong: Dang... I just bought the iFinder Pro and would have waited had I known a fully waterproof model was coming soon.....

lowrance_luke: They are close enough in frequency that they may interfere. Newer sounders have the ping speed feature which allows you to slow down the sounder to reduce or eliminate the units sinking up at depth a

ToddM: In your opinion what's the most important thing to look for when buying a sonar unit? ie power resolution,, stuff like that

StephenDawg: you mean "syncronizing" I'm assuming...

lowrance_luke: As fopr waterproof. The iFinder regular with the Aqua-bag is waterproof and floats. Some don't like the bag but it is heavy duty, protects the unit, and I like to attach to belt loop or PFD so it never has to be fished out of the lake.

lowrance_luke: SD yes

jlong: The unit won't fit in my RAM mount with the bag on it

RickLarson: I own both the X-15 and the 18, and have noticed the 18 is much faster when zooming in on a spot. How does the X-19 compare to the 18?

lowrance_luke: Overall when shopping for sounders, power is generally not an issue in freshwater as all have sufficient power to perform well. get the most resolution you can afford and of course receiver sensitive

jlong: Will the H20 be priced the same as the Pro?

lowrance_luke: ety is the main performance specification. All the power in the world doesn’t matter if the receiver is not sensitive enough to pick up the smallest differences in echo returns

ToddM: luke, that's good info, thanks

lowrance_luke: The 18 model is newer technology which includes a faster processor thus zoom differences. In addition it is only 240 vertical to the 15 350 so less screen to redraw 18 and 19 models, as with all LCX versions are identical in performance, functions, and features, only different in type and size of display

lowrance_luke: H20 will be close to or at Ifinder pro price

lowrance_luke: Are your fingers resting?????

RickLarson: Ok, so what is the difference between using a 50 and a 200 mz transducer?

jlong: How soon is soon for its release?  I'm considering returning my PRO in exchange for an H20?

TECK has left the room.

lowrance_luke: 50 is primarily designed fro salt water use. Teh power used to transmit and the wavelength of sound all provide deeper penetration and better singal resolution specifically suited for salt water.

StephenDawg entered the room.

Lone_Stone has left the room.

Shady has left the room.

Trophymuskie: this may me a dumb question, what is the ipro and H2o?

Reef_Hawg has left the room.

lowrance_luke: We do nto recommend 50 for any freshwater use with one exception. The wider cone agle allows you to track downriggers deeper and farther behind the boat for spme great lakes applications which do not

lowrance_luke: require much accuracy or resolution. Otherwise stick to 200 kHz

Shane: How water resistant are the Eagle and Lowrance units.  Will they perform under down pour conditions without having to be covered with something.  I've never had a problem, just wondering.

BUCKEYESFAN has left the room.

esoxmn entered the room.

dcates entered the room.

lowrance_luke: iFinder H20 scheduled fo late spring... I would expect april at least...maybe May at the latest

ddfenner has left the room.

jerkin has left the room.

ToddM: When using fastrack, can you get a digital depth readout in that mode?

RickLarson: Ok, I do fish deep water often. Is it a simple splice placement to use both transducers?

lowrance_luke: Ifinder pro is the updated version fo the Lowrance iFinder and the H@) version is a new waterproof model TROP

esoxmn has left the room.

Diet_Shady entered the room.

lowrance_luke: In fastrack you read depth from the scale. The digital sonar will still proces those signals for readout but thier may be a small (500 millisecond) delay

jlong: Trophymuskie, the iFinder is a handheld GPS

sworrall: that's small

ToddM: Thanks luke, where i see that coming in handy is when you have a rapid change in depth

lowrance_luke: For dual operation you will use either a dual frequncy transducer or two transdcuers on an accessory transdcuer switch avaialbe from LEI Extra's our accessory division

StephenDawg: sworrall: Yes Steve. Handheld IS small //0

Diet_Shady has left the room.

lowrance_luke: Correct Todd

Trophymuskie: Thanks jason, i guess it came out long after my lowrance 120  lol

ToddM tells you: you have to goto ohio next year

sworrall: what's on the drawing board for new units we haven't seen yet, any clues?

lowrance_luke: LFG-120 model manufactured from 72-75. Be careful you don't date yourself

Trophymuskie: I want to know what year you guys are looking at fish identification?

Reef_Hawg entered the room.

lowrance_luke: New product models are a closelt held secret in teh competitive market we are in. Be assured as the leader in this industry we will continue to bring more technology to bear at affordable prices to

lowrance_luke: you guys

Trophymuskie: I'm not that old mine was a global map i think

RickLarson: One day last year one of my units became very wet and the next morning some of the buttons wouldn't function for awhile. After it dried out it worked fine and has worked fine ever since. Do you think

RickLarson: I have anything to worry about?

lowrance_luke: Fish identification

sworrall: figured that, but had to ask!

Trophymuskie: yah bass. walleye ect you know DNA readings

H_K_: lowrance makes a great product.I still use a 20 year old paper graph on Michigan..it reads a minnow in 100 fow going 35mph..my lcd"s wont do that even on fast track...your saying the new ones will ?

sworrall: TM, sheesh!

lowrance_luke: Speciating fish is not possible with onyl technology. Understanding each species and how they relate to structure,bait, thermoclines, or other sonar features is key.

lowrance_luke: The sounder is only as good as the fisherman interpreting it

Plitz entered the room.

Shane: Is there any interference problems when using a trolling motor attached to the same battery as the Eagle/Lowrance.  If so, what is the remedy.

Trophymuskie: I can tell the differenc but the avg folks woauld like to know  ;-)

Steve_Jonesi entered the room.

ghoti has left the room.

Steve_Jonesi has left the room.

lowrance_luke: New models have more power and better sensitivety. I would match my LCX againsta any paper graph for performance, reoslution and accuracy. Paper good...LCX better

Trophymuskie: I bet before i die the finders will tell you not only the specie but exact size of the fish.

jlong: What is the waterproof rating on the old Global Map 100?

H_K_: good point!

Trophymuskie: don't drop it in the drink

Reef_Hawg: Shane, have you tried running it on the starting battery?  If there is any int. as you stated, often that corrects it.

Steve_Van_Lieshout entered the room.

ToddM: TM that might frighten you on your water!

lowrance_luke: Trolling motors developa sever current drain when activated which can effect sonar performance. We always recommend running sounders from the cranking battery. Noise and itnerference can be reduced

Shane: Thanks Reef_Hawg and Luke!

mavmskyb8 entered the room.

lowrance_luke: or eliminated using any one of teh manfuacturers grounding kits, and additionally grounding the crank battery to teh troll battery so both are at the sa,e ground potential elliminates alot of problems

sworrall: that's what I did

lowrance_luke: Global Map 100 IPX7 waterproof. Withstand immersion without water infiltration

StephenDawg: How do you like the marriage between say, Lakemaster mapping software and your high end units?

sworrall: ground wire from start to troll motor battery

Plitz: so if I'm runing a pull start motor.....would a little lawn mower battery work good to run the sonar?

Steve_Van_Lieshout has left the room.

StephenDawg: Plitz: That's what I do. It works great

Plitz: ok

StephenDawg: never had a problem at all

lowrance_luke: We promote and encourage any development of mapping data for the benefit of our fisherman. Lakemaster had compiled alot of very goos mapping details and Navionics converted to thier format

H_K_: Luke whats the LCX cost appx. sorry if already stated

Plitz: how long do you get out of a charge?

StephenDawg: Also run my handheld GPS with it  too

lowrance_luke: for Lowrance compatibility.

Reef_Hawg: Plitz.  That is how I run my sonar up front, and it works great!

StephenDawg: Nevr ran out ever

Slamr: Wow this hour went fast, I'm NOT shutting you guys down, if you want to stay Luke, please do, but our members and MuskieFIRST thank you very much for your time and knowledge!

sworrall: Thanks Luke, for a great hour!

H_K_: thanks Luke

lowrance_luke: Any 12 volt battery source will operate the sounder. Many motorcycle batteris made it onto the ice in the early days

StephenDawg: Thanks again, Luke. Best chat of the year imho.

mavmskyb8: sworrall- when will this chat be posted on the MuskiyFirst site?

Shane: Luke, Thanks for the great information!  Gotta go....take care

Slamr: Luke I know that Mr. Worrall is with me when I say if you can ever make it to one of our outtings, you always have a place in my boat, get you after some REAL fishes

Shane has left the room.

Plitz: now I'm off to get the 480 and a new batt.

lowrance_luke: Thank you gentlemen always pleasure

ToddM: thanks for the chat luke, I have always had lowrance and eagle products, no reason for me to ever change that

VMS entered the room.

Trophymuskie: wow Luke your a load of information, wish I had you in my boat for a day. //15  Thank you

Reef_Hawg: plitz, Fishmark 480?

VMS: Hi Everyone!!

Plitz: yea

StephenDawg: Plitz: pick me up an X-15 and put it on your card ole' buddy //16

lowrance_luke: Haven't boated a muskie in over 23 years... don't know if I can cast that long anymore

Slamr: we'll troll

lowrance_luke: im in

dcates has left the room.

Reef_Hawg: I came late.  I have a 320, is the 480 worth the extra?  I have a potential buyer for mine.

Trophymuskie: LOL we can troll up monsters up here  :-)

StephenDawg: lowrance_luke: ToddM will take you trolling

J_P has left the room.

sworrall: no worries, can just troll if you have to!

Plitz: it's only 200

esox-dan has left the room.

lowrance_luke: More resolution is always beneficial. Otherwise the same performance

ToddM: I can do that

Reef_Hawg: thanks guys.  I went from the old 240(optima) to this and already realize the difference.

StephenDawg: lowrance_luke: Lakemaster software. Opinions?

Plitz: Reef_Hawg: how wold ya rate the fishmark....I think they look very nice for the cash

lowrance_luke: opinion of software or Navionics distribution of thier charts?

StephenDawg: Both

lowrance_luke: Fishmark... a Luke morris best buy rating

StephenDawg: their mapping seems very thorough

sworrall: cool

Reef_Hawg: I love mine.  I have been an ultra classic user for years too, and the fishmarks are the same units with a couple added features, and much better resolution as luke stated.  Well worth it.

VMS: Question for you Luke:  If I have an x65 on the back of the boat and would like to add something to the front.  How much difference in frequncy should I be looking for between units?

Plitz: I was going to get the garmin 240 until this unit came along

Plitz: seems very well priced for what ya get

Trophymuskie: Plitz no bad words here

lowrance_luke: LakeMater software provides details not availabel from other sources and is therfore by nature beneficial. They have worked hard to produce original detailed lake data for the lakes you are most

lowrance_luke: interested in

jlong: Luck, how concerned should I be fore keeping my iFinder out of the rain and in the bag?

Reef_Hawg: no doubt.  blows the 240 away.  my buddy has one and wishes he would have waited.

lowrance_luke: We apllaud Navionics for converting that data to electronic charts and encourage them to continue to add more mapping data from lakemaster sources

StephenDawg: Reef_Hawg: I love my old Ultra Classic too. Works great

ESOX_Maniac entered the room.

H_K_ has left the room.

StephenDawg: lowrance_luke: I met two of the owners on Vermilion last summer while they were mapping it. they were very helpful and great to visit with

mavmskyb8: where can one get the Lakemaster chips?

lowrance_luke: Teh iFinder will take as much rain as you can. I had mine in the tree stand a couple of weeks ago in steady downpour and still marked the HOG when he fell on his chops

Reef_Hawg: You won't be dissapointed in the fm then.  I fish both rivers(where detail isn't all that important) and a fair share of open water stuff too, and like it for that.  Just wish it was 3000 watts inste.

StephenDawg: Even gave me a few reefs to check out

kyle entered the room.

jlong: So if I leave my iFinder in the RAM mount on the dash of my boat through an all day rain... it will be OK?

StephenDawg: mavmskyb8: Lakemaster has a website.

jlong: Keep in mind... I've had moisture issues with my Global Map 100

lowrance_luke: mav lakemaster data is avaialbe from Navionics pre-programmed on MMC card. Not all LakeMaster lakes are currently available but I am sure they are working on it

VMS: I'm gonna try this again...If I have an X65 off the transom and want to add a different unit to my bow, how much difference in frequency should I be looking for in the transducer reduce interference?

Trophymuskie: Jlong make sure it is as much under the windshild as possible and angled so the rain will run off and you ahould be ok. i never had any problems with my global map

sworrall: Luke, we will let you go anytime you wish, thanks again from MuskieFIRST

StephenDawg: their maps make Hot Spots look like kindergarten

lowrance_luke: I would say yes leaving it all day of course it depends on some contibuting factors. As a rule of thumb I would say if you would hold your cell phone in it then teh iFinder will be alright

RickLarson: Luke, is there such a thing as getting a deal on a show display X-15  ?

lowrance_luke: VMS with the ping speed adjustment you can reduce or eliminate noise and cross talk by slowing down one sounder. Free emulators are avaialbe for download from the website to see this and all new

Reef_Hawg: Too bad their GPS units are not quite as water tight as the locators.  I do love my global map 100, but have had a drop or two inside.

lowrance_luke: functions and fetures offered on our units today

jlong: I have no clue how water resistant my cell phone is.  I dropped mine in Lake of the Woods and it died.... but never tested it in the rain.

lowrance_luke: We are the only company that will let you take aunit for a test drive before making a purchasing decision

VMS: there is a ping speed adjustment on my x65?

Slamr: guys give luke a break....MuskieFIRST's insurance doesnt cover carpal tunnel for our chat guests

ESOX_Maniac: Luke- my appology for getting here  late. What are your thoughts on forward/side scanning sonars? Is Lowrance going to come out with one?

kyle has left the room.

StephenDawg: Slamr: He's typing with his fists. Can't you tell?//2

lowrance_luke: Dealeres and distributors can sell of show display models at thier discretion, they already bought them. Warranty can sometime be an issue if they are really old

ToddM: www.carpaltunnelfirst.com

lowrance_luke: VMS only on our latest models

StephenDawg: ToddM: //19

VMS has left the room.

jlong: I love my Global Map 100... that's why I bought the iFinder Pro.  I keep that thing out in rain all day and never think twice about it.  When the iFinder came with plastic bag.... I wasn't sure WHEN

Slamr: Luke we wont consider you rude if you do a "gotta go, bye" then disappear....

jlong: I needed to use it

lowrance_luke: ESOX I covered that pretty thouraly during the chat. Check out the transcript when it is posted I think it will answer much of your questions

jlong: hehehehehheh... good one slamr

ESOX_Maniac: thanks-

Reef_Hawg: My buddy bought the ifinder and wonders the same thing.

lowrance_luke: Thanks SLAMR I'm not afraid to say I've had enough when it occurs

sworrall: Hey guys, let's close off the questions now so I can archive this. GREAT session, Luke.

ESOX_Maniac: NP- ToddM- wassup?

davej entered the room.

ESOX_Maniac: Thanks Luke

Slamr: ToddM, any predictions on when the Eastern waters will open up?

sworrall: We can have you back soon, if that works for you

Reef_Hawg: can't you just archive what we have so far, and let us keep asking?  I got here late and am learnig a great deal!

jlong: Ooops... didn't realize we were still being archived as part of the chat session

sworrall: popular!!

lowrance_luke: OK "gotta go....bye.......

Trophymuskie: Thanks Luke great chat, if you ever come to eastern canada look me up  :-)

lowrance_luke has left the room.

Slamr: lol, thanks for the chat Luke!

RickLarson: Gotogo, thanks for the chat. Must say I really like my X-18 and plan to use it for a long time. Cu''s L8ter!

ToddM: i will say by middle of march if it stays like this

Reef_Hawg: Oh well.  I can't wait to see the archive on this one!  Great chat!

RickLarson has left the room.

sworrall: good one, yes!

ToddM: some great info in that chat

ESOX_Maniac: Me too- just got home & realized it was tues nite chat time- actually late.

Trophymuskie: Mr Worrall the save chat option on the bottom, will it save the entire chat for us?